# RECOMMENDED SPECIFICATION FOR CONCRETE, MORTAR BED, OR DOUBLE LAYER PLYWOOD OVER ACOUSTI-MAT® SOUND DEADENING MAT

# PART 1	GENERAL

## 1.01	SUMMARY

This is the recommended specification for concrete, mortar bed, or double layer plywood over Acousti-Mat sound deadening pad.

## 1.02	SECTION INCLUDES

- Acousti-Mat Sound Control Mats: Acousti-Mat 3/8, Acousti-Mat 3/8 Premium, Acousti-Mat 3/4, Acousti-Mat 3/4 Premium

Acousti-Mat Perimeter Isolation Strip

Acousti-Mat Tape

Maxxon Reinforcement or Reinforcement Wire (concrete or mortar bed applications)

Concrete (Option-1)

Mortar Bed (Option-2)

Double Layer Plywood (Option-3)

## 1.03	QUALITY ASSURANCE

- Acousti-Mat shall be installed according to Manufacturer’s install instructions.

- Concrete, mortar bed or double layer plywood shall be installed by an experienced applicator.

## 1.04	DELIVERY, STORAGE AND HANDLING

- General Requirements: Materials shall be delivered in their original, unopened packages, and protected from exposure to the elements. Damaged or deteriorated materials shall be removed from the premises.

## 1.05	SITE CONDITIONS

- Environmental Requirements: Before, during and after installation of concrete, mortar bed, or double layer plywood and Acousti-Mat Sound Control Mat, building interior shall be enclosed and maintained at a temperature above 50 degrees F (10 degrees C).

## 1.06	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	MATERIALS

- Acousti-Mat Perimeter Isolation Strip: Acousti-Mat Perimeter Isolation Strip as supplied by Acoustical Solutions.

Sound Deadening Mat: Acousti-Mat 3/8, Acousti-Mat 3/8 Premium, Acousti-Mat 3/4, or Acousti-Mat 3/4 Premium as supplied by:
Acoustical Solutions, 3017 Vernon Road, Richmond, VA 23228, (800) 782-5742
AcousticalSolutions.com.

Acousti-Mat Tape as supplied by:
Acoustical Solutions, 3017 Vernon Road, Richmond, VA 23228, (800) 782-5742
AcousticalSolutions.com or equal.

Maxxon Reinforcement (MR) as manufactured by Maxxon Corporation, Hamel, MN.

Reinforcement Wire: Welded wire mesh 6 inch x 6 inch or equal.

Concrete: Portland cement concrete as manufactured by local batch plant.

Mortar Bed: Portland cement mortar bed as manufactured by Bostik, Mapei, TEC, or equal.

Plywood: Plywood shall be a minimum 3/8" thick over Acousti-Mat 3/8 or Acousti-Mat 3/8 Premium, or a minimum of 3/4" thick over Acousti-Mat 3/4 or Acousti-Mat 3/4 Premium (minimum 4 ply thick, grade A, B, J, or X) that meets all the hardwood manufacturer’s specifications.

# PART 3	EXECUTION

## 3.01	PREPARATION

- Condition and Cleaning of Subfloor: Subfloor shall be structurally sound. General Contractor shall clean subfloor to remove mud, oil, grease, and other contaminating factors before the installation of the Acousti-Mat and topping.

Expansion Joints: Allow joints to continue through the concrete, mortar bed or plywood at the same width.

## APPLICATION OF ACOUSTI-MAT AND (OPTIONAL) REINFORCEMENT MESH

- Acousti-Mat and Acousti-Mat Perimeter Isolation Strip Installation: Install Acousti-Mat and Acousti-Mat Perimeter Isolation Strips following manufacturer’s recommendations and specifications.

Reinforcement Application (for Concrete or Mortar Bed topping): Maxxon Reinforcement or Reinforcement Wire: The MR or wire is loose laid and over-lapped approximately 2 inches on all sides. The thickness of the Concrete will be minimum 
2 ½ – 3 inch over the MR or wire. Do not mechanically attach the MR or wire through the Acousti-Mat to the subfloor.

## 3.03	APPLICATION OF CONCRETE, MORTAR BED OR DOUBLE LAYER PLYWOOD OVER ACOUSTI-MAT

- Scheduling: Application of concrete, mortar bed, or double layer plywood shall not begin until the building is enclosed, including roof, windows, doors, and other fenestration.

Schedule the Acousti-Mat sound control system as late as possible in the construction cycle.

Concrete Application: Place the recommended concrete a minimum 2 ½ – 3-inch-thick over Acousti-Mat. Spread and screed Concrete over the reinforcement wire to a smooth surface.

Mortar Bed Application: Place the recommended Mortar Bed a minimum 1 ¼ inch (33 mm) thick over Acousti-Mat. Spread and screed Mortar Bed over the reinforcement wire to a smooth surface.

Plywood Application: Follow the Hardwood manufacturer’s recommendations for gluing, nailing, or screwing two layers of plywood together. DO NOT nail, screw, or staple the plywood through the Acousti-Mat. The double plywood underlayment must be floating over the Acousti-Mat.

Install finished floor goods to the concrete, mortar bed, or double plywood underlayment per manufacturer’s recommendations. Care must be taken to ensure no nails, screws, or staples penetrate the Acousti-Mat.

## 3.04	PROTECTION

- Protection From Heavy Loads: During construction, place temporary wood planking over Acousti-Mat, concrete, or mortar bed wherever it will be subject to heavy wheeled or concentrated loads.

# END OF SECTION