# RECOMMENDED SPECIFICATION FOR ACOUSTI-TOP® SOUND DEADENING MAT OVER CONCRETE WITH FLOOR INSTALLATION DIRECTLY ON ACOUSTI-TOP SOUND DEADENING MAT

# PART 1	GENERAL

## 1.01	SUMMARY

This is the recommended specification for Acousti-Top Floor Underlayment over concrete with floor installation directly on Acousti-Top Sound Deadening mat.

## 1.02	SECTION INCLUDES

- Acousti-Top Adhesive (optional)

Acousti-Top Sound Deadening Mat

Acousti-Top Perimeter Isolation Strips (optional)

## 1.03	QUALITY ASSURANCE

- Acousti-Top Installer’s Qualifications: Installation of Acousti-Top shall be performed according to Maxxon installation procedures.

## 1.04	DELIVERY, STORAGE AND HANDLING

- General Requirements: Materials shall be delivered in their original, unopened packages, and protected from exposure to the elements. Damaged or deteriorated materials shall be removed from the premises.

## 1.05	SITE CONDITIONS

- Environmental Requirements: Before, during and after installation of Acousti-Top Sound Deadening Mat, building interior shall be enclosed and maintained at a temperature above 50 degrees F (10 degrees C).

## 1.06	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	MATERIALS

Optional Adhesive: Acousti-Top Adhesive as supplied by:
Acoustical Solutions, 3017 Vernon Road, Richmond, VA 23228, (800) 782-5742
AcousticalSolutions.com.

Sound Deadening Mat: Acousti-Top Sound Control Mat as manufactured by Maxxon Corporation, Hamel, MN.

Optional Acousti-Top Perimeter Isolation Strips as supplied by: 
Acoustical Solutions, 3017 Vernon Road, Richmond, VA 23228, (800) 782-5742
AcousticalSolutions.com.

# PART 3	EXECUTION

## 3.01	PREPARATION

- Condition and Cleaning of Subfloor: Subfloor shall be structurally sound. General Contractor/Project Superintendent shall clean subfloor to remove mud, oil, grease, and other contaminating factors before the arrival of the Acousti-Top crew.

Expansion Joints: Allow joints to continue through the Acousti-Top at the same width.

## APPLICATION OF ACOUSTI-TOP

- Install Acousti-Top following manufacturer’s recommendations and specifications.

## 3.03	PREPARATION FOR INSTALLATION OF THIN SET APPLIED FLOOR GOODS TO ACOUSTI-TOP

- Thin Set Applied Floor Goods Procedures: Apply a Maxxon-approved anti-fracture membrane to the Acousti-Top per manufacturer’s application recommendations. Thin set shall then be applied directly to the cured anti-fracture membrane.

- Glue Down Engineered or Laminate Wood Floor Procedure: Utilize a quality urethane wood floor adhesive to attach wood floors to Acousti-Top. Follow respective floor goods and adhesive manufacturer’s recommendations for determination and use of their urethane-based adhesives.

- Glue Down Luxury Vinyl Tile (LVT) or Luxury Vinyl Plank (LVP) Floor Procedure: Follow respective floor goods and adhesive manufacturer’s recommendations for determination and use of their urethane-based adhesives.

- Floating Wood Floor Procedure: Follow the wood floor manufacturer’s recommendations for installation of the floating wood floor system.

- Floating Luxury Vinyl Tile (LVT) or Luxury Vinyl Plank (LVP) Floor Procedure: Follow the manufacturer’s recommendations for installation of the floating LVT/LVP floor system.

## 3.04	PROTECTION

- Protection From Heavy Loads: During construction, place temporary wood planking over Acousti-Top wherever it will be subject to heavy wheeled or concentrated loads.

# END OF SECTION