(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR ALPHAPERF® V-GROOVE FACED METAL ACOUSTICAL WALL PANELS

# PART 1	GENERAL

## 1.01	SECTION INCLUDES

Sound Absorptive Wall Panels

Sound Absorptive Ceiling [Panels] [Clouds]

Sound Absorptive Ceiling Tile

Miscellaneous Accessories.

## 1.02	REFERENCES

- ASTM International:

ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

ASTM E84 Standard Test Method for Surface Burning Characteristics of Building Materials.

ASTM E795 Standard Practices for Mounting Test Specimens During Sound Absorption Tests

## 1.03	SYSTEM DESCRIPTION

- Performance Requirements:

- Surface Burning Characteristics ASTM E84 Class A

## 1.04	SUBMITTALS

- General: Submit listed submittals in accordance with Conditions of the Contract and Division 1 Submittal Procedures Section.

- Product Data: Submit product data sheet, for specified products.

- Shop Drawings: Submit shop drawings showing layout, edge profiles and panel components, including anchorage, accessories, finish colors and textures.

- Samples: Submit selection and verification samples of finishes, colors and textures.

- Test Reports: Certified test reports showing compliance with specified performance requirements

## 1.05	DELIVERY, STORAGE AND HANDLING

- General: Comply with Division 1 Product Requirements Section.

- Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

- Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by the manufacturer.

## 1.06	PROJECT CONDITIONS

- Environmental Requirements: Do not install panels until building has been enclosed and environmental conditions approximate interior conditions that will exit during occupancy.

## 1.07	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	MANUFACTURERS

- Acceptable Manufacturer:
Acoustical Solutions
3017 Vernon Road
Richmond, VA 23228
Telephone: 800-782-5742
Fax: 804-346-8808
E-mail: info@acousticalsolutions.com
Website: AcousticalSolutions.com

Substitutions: No substitutions permitted.

## 2.02	V-GROOVE FACED ACOUSTICAL WALL PANELS

- Acoustic Panels: Alphaperf Metal Absorptive Wall Panels

- Panel Thickness: 2.625 inches (66.7 mm)

- Panel Width: 30 inches (762 mm)

Panel Performance:

- Sound Absorption: Provide panels that are certified to meet the following minimum sound absorption coefficients when tested in accordance with ASTM C423 and E795:

- Flush Mount:
125 Hz:	0.30
250 Hz:	0.71
500 Hz:	1.21
1000 Hz:	1.31
2000 Hz:	1.22
4000 Hz:	1.14
NRC:		1.10 minimum

Stand-Off Mount:
125 Hz:	0.56
250 Hz:	1.11
500 Hz:	1.33
1000 Hz:	1.31
2000 Hz:	1.31
4000 Hz:	1.23
NRC:		1.25 minimum

- Fire: Provide panels when tested in accordance with ASTM E84 demonstrate:
Flame Spread = 10
Smoke Developed=10

Panel Construction:

- All materials to be electro galvanized / galvannealed steel.

- Panel body shall be fully formed one piece perforated metal shell. Material shall be minimum 22 gauge (.76 mm) steel sheet perforated with 3/32 inch (2 mm) diameter holes on 5/32 inch (4 mm) staggered centers for a 33% open area. V-Groove ridges shall be formed on 6 inch (152 mm) centers x 0.625 inches (16 mm) deep.

- Internal framing and reinforcement s when required shall be a minimum of 20 gauge (0.9mm) steel channels spotwelded and / or riveted to the panel body shell.

Acoustical Insulation: Fill shall be 2 inch (50 mm) thick fiberglass or mineral wool having a density of not less than 1.5 pounds per cubic foot (24 kg/cubic m). When required, panel fill shall be totally encased in a 2 mil thick heat sealed black poly wrap

Finish: Manufacturer’s standard acrylic enamel or baked polyester powder coat.

Panel Mounting: Manufacturer’s standard minimum 14 gauge (1.90 mm) brackets, four per cloud panel.

# PART 3	EXECUTION

## 3.01	EXAMINATION

- Examine surfaces to receive acoustical clouds. Do not begin installation until any unsatisfactory conditions have been corrected.

## INSTALLATION

- Install clouds on ceiling in locations and in patterns as indicated on the drawings.

Install each cloud in accordance with manufacturer’s instructions using approved anchors and fasteners.

## 3.03	CLEANING

- Follow manufacturer’s instructions for cleaning clouds soiled during installation. Replace clouds that cannot be cleaned to as new condition.

- Keep site free from accumulation of waste and debris.

# END OF SECTION