(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR 
ALPHASORB® BARRIER FABRIC WRAPPED ACOUSTIC PANELS

# PART 1	GENERAL

## 1.01	SUMMARY

This section includes the following:

Acoustical Wall Panel

## 1.02	SUBMITTALS

- Shop Drawings: Show fabrication and installation details for acoustical wall panels, including plans, elevations, sections, details and attachment to other work.

Submittals: Furnish 4” x 6” samples, including color chart showing all of the manufacturer’s full range of colors, texture and patterns available for each type of acoustical product specified.

Product Test Report: From a qualified testing agency indicating wall panels comply with requirements.

Qualification Data: For firms specified in “Quality Assurance” article to demonstrate their capabilities and experience.

## 1.03	QUALITY ASSURANCE

Note to Specifier: Paragraph below should list obligations for compliance with specific

code requirements particular to this section. General statements to comply with a

specific code are typically addressed in Conditions of the Contract and Division 1

Regulatory Requirements Section. Repetitive statements should be avoided.

- Manufacturer Qualification: Manufacturer shall have a minimum of 5 years’ experience in production of specified products and shall furnish supporting documentation showing completed jobs of approximately the same size and scope.

Fire Test Reports: Provide acoustical wall panels with the following surface-burning characteristics as per ASTM E 84.

- Flame Spread: 25 or less

Smoke Developed: 450 or less

Acoustical Test Report: Provide acoustical test report from a qualified testing agency indicating acoustical wall panels meets 0.75 NRC per ASTM C-423.

## 1.04	DELIVERY, STORAGE AND HANDLING

- Protect Acoustical Wall Panels from excessive moisture when shipping, storing, and handling. Deliver in unopened skids and store in a dry place with adequate air circulation. Do not delivery material until all wet-work has been completed.

## 1.05	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	WALL PANELS, GENERAL

- Acoustical Wall Panels shall be 1⅛" thick. Panel shall be constructed of 2 layers of ½" thick 6-7 lbs. per cubic foot high-density fiberglass laminated in between a 1 lb. per square foot mass loaded vinyl barrier having a minimum STC rating of 29. Manufacturer’s standard panel construction, fully wrapped panel. Manufacturer’s standard full line of fabric.

Products

- Model # AlphaSorb® Barrier wall panel

Manufacturer:
Acoustical Solutions, LLC

- 3017 Vernon Road, Richmond, VA 23228
Telephone: (800) 782-5742
Fax (804) 346-8808

Facing Material: Guilford of Maine FR 701 – Style 2100 – 100% polyester

Panel Thickness: nominal 1⅛" thick

Panel Weight: 1.75 lbs. per square foot

Sound Transmission Coefficient: 29

Noise Reduction Coefficient: NRC 0.75

Maximum Panel Width: 48 inches

Maximum Panel Length: 120 inches

## 2.02	CONSTRUCTION

- The acoustical wall panel product shall be supplied in desired width and the desired lengths as indicated. All edges will be fully wrapped with mounting as indicated.

## 2.03	MOUNTING

- Back-Mounting Accessories: Manufacturer’s standard accessories for securely mounting panels, of type and size indicated and complying with the following requirements:

- Mechanically Mounted Edge-Reinforced Panels: Metal impaling clips designed to support weight of panels, mechanically attached to wall substrate according to manufacturer’s standard pattern and adhesively bonded to back of panel, with base support brackets where recommended by manufacturer for additional support of panels.

## 2.04	FLAMMABILITY RATING

- All components shall have a Class A Flammability rating per ASTM E-84: Surface Burning Characteristics of Building Materials, with a Flame Spread of 25 or less and Smoke Developed of 450 or less.

# PART 3	EXECUTION

## 3.01	INSTALLATION

- Install Acoustical Wall panels in locations indicated, top edges level and in alignment with other panels. Comply with manufacturer’s written instructions for installation of panels using type of mounting accessories indicated or, if not indicated, as recommended by manufacture.

## 3.02	CLEANING

- After completion of installation of panels, remove dust and other foreign material according to manufacturer’s written instructions.

Remove surplus material, rubbish, and debris resulting from panel installation, on completion of the work, and leave areas of installation in a neat and clean condition.

# END OF SECTION