(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR
ALPHASORB® FABRIC WRAPPED ACOUSTIC CEILING CLOUD

# PART 1	GENERAL

## 1.01	SUMMARY

This section includes the following:

Acoustical Suspended Ceiling Cloud Mount Panel

## 1.02	SUBMITTALS

- Shop Drawings: Show fabrication and installation details for acoustical cloud panels, including plans, elevations, sections, details and attachment to other work.

Submittals: Furnish 4" x 6" sample, color chart showing all manufacturer’s full range of colors, texture and patterns available for each type of acoustical product specified.

Product Test Report: From a qualified testing agency indicating wall panels comply with requirements.

Qualification Data: For firms specified in “Quality Assurance” Article to demonstrate their capabilities and experience.

## 1.03	QUALITY ASSURANCE

Note to Specifier: Paragraph below should list obligations for compliance with specific

code requirements particular to this section. General statements to comply with a

specific code are typically addressed in Conditions of the Contract and Division 1

Regulatory Requirements Section. Repetitive statements should be avoided.

- Manufacturer Qualification: Manufacturer shall have a minimum of 5 years’ experience in production of specified products and shall furnish supporting documentation showing completed jobs of approximately the same size and scope.

Fire Test Reports: Provide acoustical cloud panels with the following surface- burning characteristics as per ASTM E 84.

- Flame Spread: 25 or less

Smoke Developed: 450 or less

Acoustical Test Report: Provide acoustical test report from a qualified testing agency indicating acoustical cloud panels meets 0.85 or 1.10 NRC per ASTM C-423.

## 1.04	DELIVERY, STORAGE AND HANDLING

- Protect Acoustical Cloud Panels from excessive moisture when shipping, storing, and handling. Deliver in unopened skids and store in a dry place with adequate air circulation. Do not delivery material until all wet-work has been completed.

## 1.05	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	CLOUD PANELS, GENERAL

- Acoustical Cloud Panels shall be 6.0–7.0 lb. per cubic foot high-density fine fibered fiberglass. Manufactures standard panel construction, wrapped with panel manufacturer’s standard full line of fabric.

- Products

AlphaSorb® Fabric Wrapped Acoustic Ceiling Cloud
Acoustical Solutions, LLC 
3017 Vernon Road 
Richmond, VA 23228
Ph. (800) 782-5742
AcousticalSolutions.com

Facing Material: [Guilford of Maine FR 701 Style 2100 100% polyester] [Guilford of Maine Anchorage Style 2335] [SoundSuede™] [Guilford of Maine Studio 54] [Guilford of Maine Whisper Fabric] or [as specified]

Panel Thickness: nominal [1"] or [2"] thick

Noise Reduction Coefficient: NRC 0.85, 1.10

Panel Width: As indicated, up to 4 feet wide

Panel Length: As indicated, up to 8 feet long

## 2.02	CONSTRUCTION

- The acoustical cloud panel product shall be supplied in widths up to 4 feet wide and lengths up to 8 feet long. All edges will be fully wrapped with mounting as indicated.

## 2.03	MOUNTING

- Back-Mounting Rotofast Cloud Anchors: Manufactures standard accessories for securely mounting panels, of type and size indicated and complying with the following requirements:

- Suspension provision: Secure Rotofast Cloud Anchors. Attach anchors to panel core in accordance with manufacturer’s instructions and suspend from the ceiling with cabling or tie wraps as necessary.

## 2.04	ACOUSTICAL PERFORMANCE

NOTE TO SPECIFIER — NRC sound absorption data is based on testing with Guilford FR701 fabric. To maintain specified sound absorption, fabrics should be selected that allow air to pass easily through the material into the acoustical core.

Sound Absorption: Per ASTM C – 423

Sound Absorption Coefficient per Octave Band Center Frequency (Hz)

125	250	500	1000	2000	4000	NRC

0.14	0.27	0.80	1.11	1.14	1.14	0.85

0.40	0.74	1.24	1.25	1.23	1.14	1.10

## 2.05	FLAMMABILITY RATING

- All components shall have a Class A Flammability rating per ASTM E-84: Surface Burning Characteristics of Building Materials, with a Flame Spread of 25 or less and Smoke Developed of 450 or less.

# PART 3	EXECUTION

## 3.01	INSTALLATION

- Install Acoustical Cloud panels in locations indicated, top edges level and in alignment with other panels. Comply with manufactures written instructions for installation of panels using type of mounting accessories indicated or, if not indicated, as recommended by manufacturer.

## CLEANING

- After completion of installation of panels, remove dust and other foreign material according to manufacturer’s written instructions.

- Remove surplus material, rubbish, and debris resulting from panel installation, on completion of the work, and leave areas of installation in a neat and clean condition.

# END OF SECTION