(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR 
ALPHASORB® BARRIER FABRIC WRAPPED ACOUSTIC PANELS

# PART 1	GENERAL

## 1.01	SUMMARY

This section includes the following:

Acoustical Wall Panel

## 1.02	SUBMITTALS

- Shop Drawings: Show fabrication and installation details for acoustical wall panels, including plans, elevations, sections, details, and attachment to other work.

Submittals: Furnish 12" x 12" samples, showing manufacture full range of colors, texture and patterns available for each type of acoustical product specified.

Product Test Report: From a qualified testing agency indicating wall panels comply with requirements.

Qualification Data: For firms specified in “Quality Assurance” article to demonstrate their capabilities and experience.

Product Certificates: Signed by manufacturer certifying that the products furnished comply with requirements.

## 1.03	QUALITY ASSURANCE

Note to Specifier: Paragraph below should list obligations for compliance with specific

code requirements particular to this section. General statements to comply with a

specific code are typically addressed in Conditions of the Contract and Division 1

Regulatory Requirements Section. Repetitive statements should be avoided.

- Manufacturer Qualification: Manufacturer shall have a minimum of 5 years’ experience in production of specified products and shall furnish supporting documentation showing completed jobs of approximately the same size and scope.

Fire Test Reports: Provide acoustical wall panels with the following surface-burning characteristics as per ASTM E 84.

- Flame Spread: 25 or less

Smoke Developed: 450 or less

Acoustical Test Report: Provide acoustical test report from a qualified testing agency indicating acoustical wall panels meets [0.70, 0.90, 0.95] NRC per ASTM C-423.

## 1.04	DELIVERY, STORAGE AND HANDLING

- Protect Acoustical Wall Panels from excessive moisture when shipping, storing, and handling. Deliver in unopened skids and store in a dry place with adequate air circulation. Do not delivery material until all wet-work has been completed.

## 1.05	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	WALL PANELS, GENERAL

- Acoustical Wall Panels shall be [⅝", 1⅛", 2⅛"] thick dual density core, specifically designed to support the Signature paintable facing. Manufacturer’s standard panel construction with fully wrapped edge.

Products

- Model # AlphaSorb® Paintable Acousti Panel

Manufacturer:
Acoustical Solutions, LLC

- 3017 Vernon Road, Richmond, VA 23228
Telephone: (800) 782-5742
Fax (804) 346-8808

Facing Material: Signature White Paint-ready fiber veil

Panel Thickness: nominal [⅝", 1⅛", 2⅛"] thick

Noise Reduction Coefficient: NRC [0.70, 0.90, 0.95]

Panel Width: 48 inches

Panel Length: As indicated, up to 120 inches

## 2.02	CONSTRUCTION

- The acoustical wall panel product shall be supplied in 48" wide panels with lengths as indicated. All edges will be fully wrapped with mounting as indicate.

## 2.03	MOUNTING

- Back-Mounting Accessories: Manufacturer’s standard accessories for securely mounting panels, of type and size indicated and complying with the following requirements:

- Mechanically Mounted Edge-Reinforced Panels: Metal clips designed to support weight of panels, mechanically attached to wall substrate according to manufacturer’s standard pattern and adhesively bonded to back of panel, with base support brackets where recommended by manufacturer for additional support of panels.

## 2.04	ACOUSTICAL PERFORMANCE

- SOUND ABSORPTION PER ASTM C-423
Sound Absorption Coefficient per Octave Band Center Frequency (Hz) 
125	250	500	1000	2000	4000	NRC
		Per Manufacturer’s Specifications

## 2.05	FLAMMABILITY RATING

- All components shall have a Class A Flammability rating per ASTM E-84: Surface Burning Characteristics of Building Materials, with a Flame Spread of 25 or less and Smoke Developed of 450 or less.

# PART 3	EXECUTION

## 3.01	INSTALLATION

- Install Acoustical Wall panels in locations indicated, top edges level and in alignment with other panels. Comply with manufacturer’s written instructions for installation of panels using type of mounting accessories indicated or, if not indicated, as recommended by manufacture.

## 3.02	CLEANING

- After completion of installation of panels, remove dust and other foreign material according to manufacturer’s written instructions.

Remove surplus material, rubbish, and debris resulting from panel installation, on completion of the work, and leave areas of installation in a neat and clean condition.

# END OF SECTION