(Attention Specifiers, all areas in red require you to make selection based on requirements of the project before issuing specification. Please refer to the appropriate product data page when making your selection. All data pages are available at https://acousticalsolutions.com/)

# RECOMMENDED SPECIFICATION FOR ALPHASORB® RIGID ACOUSTIC BAFFLES

# PART 1	GENERAL

## 1.01	SUMMARY

Section includes:

AlphaSorb® Rigid Acoustic Baffles.

## 1.02	SUBMITTALS

- Shop Drawings: Show fabrication and installation details for ceiling baffles, including plans, elevations, sections, details and attachment to other work.

Submittals: Furnish 8 1/2” x 11” samples, showing manufacture full range of colors, texture and patterns available for each type of acoustical baffle specified.

Qualification Data: For firms specified in “Quality Assurance” Article to demonstrate their capabilities and experience.

Product Certificates: Signed by manufacturer certifying that the products furnished comply with requirements.

## 1.03	QUALITY ASSURANCE

Note to Specifier: Paragraph below should list obligations for compliance with specific

code requirements particular to this section. General statements to comply with a

specific code are typically addressed in Conditions of the Contract and Division 1

Regulatory Requirements Section. Repetitive statements should be avoided.

- Performance Requirements

- Manufacturer Qualification: Manufacture shall have a minimum of 5 years experience in production of specified products and shall furnish supporting documentation showing completed jobs of approximately the same size and scope.

Fire Test Reports: Provide ceiling baffles with the following surface- burning characteristics as per ASTM E-84.

Acoustical Test Report: Provide acoustical test report from a qualified testing agency indicating ceiling baffles meets specified NRC values per C423-90a.

## 1.04	DELIVERY, STORAGE AND HANDLING

- Protect Ceiling baffles from excessive moisture when shipping, storing, and handling. Deliver in unopened skids and store in a dry place with adequate air circulation. Do not delivery material until all wet-work has been completed.

## 1.05	WARRANTY

- One-year limited warranty from invoice date for defects in workmanship or material.

Warranty excludes normal wear and tear, misuse, improper installation, and external damage.

Extended warranties are available for designated products.

# PART 2	PRODUCTS

## 2.01	CEILING BAFFLES, GENERAL

- Ceiling baffles shall be 2" thick – 6 lb. per cubic foot density fiberglass. Manufacturer’s standard baffle construction, wrapped edges, baffle manufacturer’s standard full line of fabric

- Products

Model # R-100
Acoustical Solutions
3017 Vernon Rd.
Henrico, VA 23228
Ph: (800) 782-5742
Fax: (804) 346-8808
Web: acousticalsolutions.com

Facing Material: Guilford of Maine (standard)

Baffle Thickness: nominal 2" thick

NRC Value: 1.15

Baffle Width: 48 inches

Baffle Length: As indicated, up to 10 feet long. (standard 2'x4')

## 2.02	CONSTRUCTION

- AlphaSorb® Rigid Acoustic Baffle product shall be supplied in 48" wide baffles with lengths as indicated. All edges will be wrapped and all baffles come with mounting brackets.

## 2.03	MOUNTING

- Manufacturer’s standard accessories for securely mounting baffles, of type and size indicated and complying with the following requirements:

- Brackets shall be installed to facilitate mounting on either the short edge or long edge of the baffle, as indicated on the drawing.

## 2.04	ACOUSTICAL PERFORMANCE

- SOUND ABSORPTION PER ASTM C-423
Sound Absorption Coefficient per Octave Band Center Frequency (Hz) 
125	250	500	1000	2000	4000	NRC
0.22	0.81	1.24	1.30	1.21	1.16	1.15

## 2.05	FLAMMABILITY RATING

- Guilford facing material shall have a Class A Flammability rating per CPAI-84: Substrate/core shall have a Class A Flammability rating per ASTM E-84.

# PART 3	EXECUTION

## 3.01	INSTALLATION

- Install Ceiling baffles in locations indicated, top edges level and in alignment with other baffles. Comply with manufacturer’s written instructions for installation of baffles using type of mounting accessories indicated or, if not indicated, as recommended by manufacturer.

## CLEANING

- After completion of installation of baffles, remove dust and other foreign material according to manufacturer’s written instructions.

Remove surplus material, rubbish, and debris resulting from baffle installation, on completion of the work, and leave areas of installation in a neat and clean condition.

# END OF SECTION